# Specifications for Geodetic IV Antenna Body, TNC

| ASHTECH P/N | 701975-01 |  |  |  |
| --- | --- | --- | --- | --- |
| DESCRIPTION | GEODETIC IV, TNC connector |  |  |  |
| FREQUENCY | 1575 ± 10.23 MHz, 1227.60 ± 10.23 MHz |  |  |  |
| POLARIZATION | RIGHT HAND CIRCULAR |  |  |  |
| AXIAL RATIO | 3dB MAX |  |  |  |
| RADIATION COVERAGE: @ 1575.42Mhz | dBiC |  |  | = 0° |
|  | dBiC (TBD by ATT) | 0° < |  | < 75° |
|  | dBiC (TBD by ATT) | 75°  |  | < 80° |
|  | dBiC (TBD by ATT) | 80°  |  | < 85° |
|  | dBiC (TBD by ATT) |  |  | = 90° |
| AMPLIFIER: |  |  |  |  |
| GAIN | 38 dB ± 3dB |  |  |  |
| NOISE FIGURE | 2.5 dB MAX |  |  |  |
| IMPEDANCE | 50 OHMS |  |  |  |
| VSWR | 2.0:1 |  |  |  |
| BAND REJECTION | 70dB or better @ 1625Mhz |  |  |  |
| POWER HANDLING | 5-15V, 60 mA TYP or better |  |  |  |
| FINISH | Radome: White Zenoy plastic Body: Blue Zenoy plastic Groundplane: Aluminum |  |  |  |
| CONNECTOR | Female TNC |  |  |  |
| WEIGHT | 1lb 12 oz MAX |  |  |  |
| ALTITUDE | 20,000 ft (6096 m) |  |  |  |
| OPERATING TEMP. | -40C to +65C or better |  |  |  |
| STORAGE TEMP. | -55C to +75C or better |  |  |  |
| MOISTURE RESISTANCE | Submersible (passes IPX7 standard) |  |  |  |
| SHOCK | Survives 2.0 meter pole drop on to concrete. |  |  |  |
| VIBRATION | DO160C (unspecified, TBD by ATT) |  |  |  |

ANTENNA ID        DESCRIPTION                  DATA SOURCE (# OF TESTS) YR/MO/DY

|AVE = # in average

[north]  [ east]  [  up ]                                   | L1 Offset (mm)

[90]  [85]  [80]  [75]  [70]  [65]  [60]  [55]  [50]  [45]  | L1 Phase at

[40]  [35]  [30]  [25]  [20]  [15]  [10]  [ 5]  [ 0]        | Elevation (mm)

[north]  [ east]  [  up ]                                   | L2 Offset (mm)

[90]  [85]  [80]  [75]  [70]  [65]  [60]  [55]  [50]  [45]  | L2 Phase at

[40]  [35]  [30]  [25]  [20]  [15]  [10]  [ 5]  [ 0]        | Elevation (mm)

ASH701975_01            without groundplane

-2.1      -4.1      56.4

.0   1.0   2.0   3.1   4.2   5.1   5.9   6.5   6.9   7.1

7.0   6.6   5.9   5.0   3.8   2.4    .7    .0    .0

-1.7      -3.2      62.2

.0   -.3   -.2    .2    .8   1.5   2.3   2.9   3.3   3.6

3.7   3.4   2.8   2.0    .8   -.7  -2.5    .0    .0

rms -  3  measurements

.4       1.0        .4

.0    .1    .1    .1    .0    .0    .0    .1    .1    .1

.1    .1    .0    .0    .1    .2    .4    .0    .0

.6        .6        .4

.0    .1    .1    .1    .1    .1    .1    .1    .0    .0

.0    .0    .0    .0    .0    .0    .0    .0    .0