Release Notes

----------

MobileMapper Office version 4.6

TABLE OF CONTENTS

1.	Context and purpose	3

2.	Versions	3

3.	Applicable documents	3

4.	Released files location	3

5.	Installation procedure	3

6.	New features	4

7.	Missing features	4

8.	Fixed issues	4

9.	Known issues	5

# Context and purpose

This document contains the Release Notes for MobileMapper Office 4.6 to be released with MM20

# Versions

MobileMapper Office : 4.6.2

Languages	 : English, French, Spanish, German, Simplified Chinese (RPC), Japanese, Portuguese,
 Traditional Chinese (Taiwan), Korean, Italian, Greek, Polish, Czech, Hungarian, Romanian, Russian, Bulgarian, Turkish, Lithuanian, Norwegian, Swedish, Finnish

# Installation procedure

Launch Setup.exe  file and follow instructions

You can re launch applications using Desktop Icon or Windows Start Menu

# New features

Implement following mandatory requirement

- fix issue with new raw-data flagging and any encountered issue with MM20 raw-data

- Improve coordinate selection when creating new job from shape file

# Fixed issues

## Between 4.6.1 and 4.6.2

FIX Bug MMO-2: It is not possible to process a lot of maps

FIX Bug MMO-3: It is impossible to install Lithuanian version

FIX Bug MMO-5: Language list contains changeable string. (Lithuanian)

Fix FNC85: Fix crash encountered when trying to process FNC data on some computers. 
About the missing epoch in post-processing results, Igor Artushkin reported:

“The problem is in nearest base station LIL2 raw data: - we have two bad epochs inside corresponding RINEX file: at 13:36:30 – epoch with only two GLONASS SV and at 13:37:00 – epoch with 4 GPS and 5 GLONASS SV.

Because we have to interpolate base data from 30 seconds to 1 second interval – and we use the same algorithm of interpolation as NGS one – and single missed epoch in base data lead to loss of interpolated SV data for 8 epoch (i.e. 270 seconds of raw data in our case – this time span contains 4 points without any solution due to missed interpolated base data).

The solution of this problem – utilize data from other station in MMO post processing.”

## Between 4.6.0 and 4.6.1

Fix FNC68: User should be able to open DBF file using numeric field of 20 digits 
(But we can keep current restriction when creating new DBF File)

## Between 4.5.1 and 4.6.0

None

# Known issues

Bug MMO-4: [LOC] Application starts in minimize mode.