Firmware Release Notice	OEM Solutions

Date:		July 10, 2002

Product:	Magellan G12 Receiver, G12 Sensor, GG24 Eurocard, GG24 Sensor and G12E Sensor.

Subject: 	Firmware Version GM06

Number: 	G12/GG24 2001

Introduction:

GM06 is a no-cost firmware upgrade for all versions of the Magellan G12 OEM receiver board, G12 Sensor, GG24 Eurocard, GG24 Sensor and G12E Sensor to replace GM00 and older versions of firmware.  This is the first release of high memory firmware for the GG24 Sensor and G12E.  Loading high memory firmware into a receiver with normal memory renders the receiver unusable and may require the unit to be returned to the factory for non-warranty repair.  To obtain a high memory firmware upgrade, contact Technical support via EMAIL or telephone with the receiver serial number and part number.  Send EMAIL to oemsupport@magellangps.com.

# 1. FIRMWARE CHANGES AND IMPROVEMENTS

- Occasional receiver reset problem in the high memory G12 was fixed.

- Occasional PRC outage problem corrected.

- The extended memory versions with the “I” option support RAIM.

- The station ID field and the satellite number fields in GGA, RRE, GSV and GNS messages were modified to meet the NMEA 3.0 requirements.

# 2. NEW COMMANDS/FEATURES (G12 and G12 Sensor only)

1. New MST binary message was added .

Function:  To output condensed MST binary message with computed co-ordinates, velocities, error estimates, HDOP, VDOP, TDOP, etc.

Command:  $PASHS,RAW,MST, c1,s2[,f3]

Enable/disable raw MST response message on output port c1, where s2 is ON or OFF, and f3 is optional numeric output interval (range is 0.05 to 999 sec depending on the raw data update rate option installed). MST message is output immediately after power up even if no satellites are tracked. MST message will be of fixed length all the time regardless of the number of satellites tracked or position availability.

Command:  $PASHQ,MST[,c1]

This is the associated query command, where c1 is the optional port designator for message output.

Format:   $PASHR,MST,<Binary Data String + checksum>

$PASHR,MST Data String

| Word/Bit | 15 |  | 14 |  |  | 13 |  |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  |  | 7 |  | 6 |  |  | 5 |  | 4 |  |  | 3 |  |  |  | 2 |  | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DN 1 | GPS Milliseconds of Week in UTC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 2 | GPS Milliseconds of Week in UTC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 3 | A |  | D | N |  |  |  | F |  |  | M |  | P |  | B |  | I |  | S |  |  |  | R4 |  | R3 |  |  | R2 |  |  | R1 |  | Fix Status |  |  |  |  |
| DN 4 | TDOP |  |  |  |  |  |  |  |  |  |  |  | HDOP |  |  |  |  |  |  |  |  |  |  |  | VDOP |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 5 | Latitude |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 6 | Latitude |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 7 | Longitude |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 8 | Longitude |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 9 | Altitude MSL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 10 | Altitude MSL |  |  |  |  |  |  |  | GPS SV Range Errors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 11 | Ground Speed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 12 | Ground Track |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 13 | East Velocity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN 14 | North Velocity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN15 | Up Velocity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DN16 | SV32 | SV31 |  |  | SV30 |  | SV29 |  |  | SV28 |  | SV27 |  | SV26 |  | SV25 |  | SV24 |  |  | SV23 |  |  | SV22 |  | SC21 |  |  | SV20 |  |  | SV19 |  |  | SV18 |  | SV17 |
| DN17 | SV16 | SV15 |  |  | SV14 |  | SV13 |  |  | SV12 |  | SV11 |  | SV10 |  | SV9 |  | SV8 |  |  | SV7 |  |  | SV6 |  | SV5 |  |  | SV4 |  |  | SV3 |  |  | SV2 |  | SV1 |

| Word | Description | Definition |
| --- | --- | --- |
| 1 & 2 | GPS Time of Week | UTC Corrected Milliseconds |
| 3 | Altitude Invalid (A) | = 1 if standard deviation of altitude error exceeds 25 ft. |
| 3 | Differential Correction flag (D) | = 0 Stand-Alone = 1 Differential Corrections Applied |
| 3 | Nav Degraded Indication (N) | Navigation Quality Degraded = 1 if standard deviation of latitude error or standard deviation of longitude error or standard deviation of altitude error exceeds 25 ft. |
| 3 | Receiver Fault Indication (F) | = 1 if GPS Receiver indicates one of these faults: - Receiver clock offset > 100 msec - BIT check Failed (turn-on “MEM” test) - Bad keep-alive battery If G12 does not support any of above tests, leave this bit as a ‘0’. If set to 1, keep latched until next cold boot, then re-verify and reset only if none of the conditions exists. |
| 3 | Almanac Old (M) | = 1 if: - No Almanac - Almanac is older than 6 months Reset when a new valid almanac becomes available. |
| 3 | GPS Receiver POR Executed (P) | = 1 if Data Link initiated GPS Receiver power on reset. Typically this bit is set if there exists a board fault, comm fault (either end), low power, or command has caused any aircraft instrumentation to reboot. Thus, at boot time, (caused by the reset line on the G12 or by normal boot process) this bit shall be set to a "1" for a single "MSTCS" message, subsequent MSTCS messages shall have this bit as a “0”. |
| 3 | BIT Status (B) | = 0 Processor passed BIT = 1 Processor failed BIT Always “0” as G12 does not support BIT for processor. |
| 3 | IODE Mismatch (I) | = 1 IODE mismatch between available ephemeris and DGPS corrections. If valid Type 2 data is available for bridging, the flag is NOT raised. |
| 3 | Antenna or GPS RF Signal Loss (S) | 1= GPS Antenna Failure (No current or no RF signal). As G12 does not support monitoring of antenna current consumption, this bit shall be set to a “1”, when zero satellites are tracked. |
| 3 | R4 R3 R2 R1 Number of pseudo ranges used in current calculation | 0000 = 0 SV range measurements used 1100 = 12 SV range measurements used (Same value as in POS message) |
| 3 | Bit 2-0 Fix Status (F) | 000=No Fix 001=DR only solution 010=Stand Alone 2D 011=Stand Alone 3D 100=Differential 2D Fix 101=Differential 3D Fix |
| 4 | TDOP | 0 to 32, lsb = 1 11111 Binary = 31 00000 Binary = 0 |
| 4 | HDOP | 0 to 32, lsb = 1 11111 Binary = 31 00000 Binary = 0 |
| 4 | VDOP | 0 to 64, lsb = 1 111111 Binary = 63 000000 Binary = 0 |
| 5 & 6 | Latitude | +/- 90 Degrees Latitude, Signed Binary 7FFF FFFFH = +90 Deg 0000 0001H = +0.00015 sec 0000 0000H = 0 Deg FFFF FFFFH = -0.00015 sec 8000 0000H = -90 Deg |
| 7 & 8 | Longitude | +/- 180 Degrees Longitude, Signed Binary 7FFF FFFFH = +180 Deg 0000 0001H = +0.0003 sec 0000 0000H = 0 Deg FFFF FFFFH = -0.0003 sec 8000 0000H = -180 Deg |
| 9 & 10 | Altitude | -2000 Feet MSL to + 63535 ft @ 0.125 Ft resolution, Unsigned (offset) Binary 7FFFFH = +63535.0 Ft MSL 03E80H = 0.0 Ft MSL 00001H = -1999.875 Ft MSL 00000 = -2000.0 Ft MSL |
| 10 | GPS SV Range Errors | Measure of GPS accuracy as RMS error. Postfit chi-square values used for comparison against the statistical values for the RAIM alarm flag shall be used to indicate GPS accuracy. Chi-square values shall be scaled into "feet of error". Scales 0 to 2048 ft. 0.25 lsb. 1FFFH = 2048 ft error 0000H = 0 ft error |
| 11 | Ground Speed | 0 - 6553.6 Feet Per Second Unsigned Binary FFFFH = 6553.5 FPS FFFEH = 6553.4 FPS 0001H = 0.1 FPS 0000H = 0 FPS |
| 12 | Ground Track (North = 0 degrees) (East = 90 degrees) (South = 180 degrees) (West = 270 degrees) | 0-359.99 Degrees, Unsigned Binary FFFFH = 359.9999 Deg FFFEH = 359.9945 Deg 0001H = 0.00549 Deg 0000H = 0 Deg |
| 13 | East Velocity | +/- 3276.8 Feet Per Second, Signed Binary 7FFFH = +3276.8 FPS 0001H = +0.1 FPS 0000H = 0 FPS FFFFH = -0.1 FPS 8000H = -3276.8 FPS |
| 14 | North Velocity | +/- 3276.8 Feet Per Second, Signed Binary 7FFFH = +3276.8 FPS 0001H = +0.1 FPS 0000H = 0 FPS FFFFH = -0.1 FPS 8000H = -3276.8 FPS |
| 15 | Up Velocity | +/- 3276.8 Feet Per Second, Signed Binary 7FFFH = +3276.8 FPS 0001H = +0.1 FPS 0000H = 0 FPS FFFFH = -0.1 FPS 8000H = -3276.8 FPS |
| 16-17 | SV32 … SV1 ID of individual SVs being used in current solution | A "1" indicates the SV is being used in the solution. A "0" indicates the SV is not being used in the solution |

# 3. Known Problems and Limitations

      - ION setting under the PAR message always indicates “Y” even during differential operation when ION is automatically set to “N”.

      - Response to $PASHQ,CLK contains an extra nuisance data field at the end before the checksum.