HOW TO USE JOINFILE

JOINFILE.EXE is a stand-alone Reliance utility which may be used in conjunction with Solutions software to join consecutive raw B files.

Note: The related E files must also be present in the same folder.

1. Copy JOINFILE.EXE to a folder on the PC hard drive. Create a Shortcut, if desired.

2. Start JOINFILE.EXE by double-clicking the name in Windows Explorer, or double-clicking a Shortcut on the Desktop.

3. Select the appropriate file format for the data, Ashtech B file or RINEX.

4.  Click on "Add File to List". A Windows dialog box opens. Browse to where the data files are stored, highlight the first B file to be joined, and click on "Open". This file name populates the first line of the Join Files dialog box.

5. Click on "Add File to List". Repeat steps in #4. Continue until all files to be joined are listed in the Join Files dialog box.

6. Click on "Join Files in List".

7. "Options" allows the used to set the maximum gap allowed between files. Large gaps (over 120 seconds) will result in lost data.

8. By default the resultant joined file will have the extension *.NEW, which must be changed now, or renamed later to agree with Ashtech B file naming format.

Note: One of the E files must also be renamed to corform to the joined B file name.