Software Release Notice

Date:		May 18,1999
Product:  	Receiver Communication Software

Subject: 		RCS v 3.2 with Download v 3.2

Number: 	RCS9901

Introduction:

1.	RCS v.3.2 is a no charge software upgrade from v.2.0. All utilities in RCS are now EOW/Y2K compliant. RCS supports Win95/98 and Win NT 4.0 (or later).

2. 	It is mandatory that RCS v 3.2 be used to upload the UF00 firmware for Z-Surveyor, Z-FX, Z Eurocard, Z-Sensor. This new firmware is not compatible with earlier versions of RCS.

New RCS Functionality

1.	A new version of Download has been developed to support the new Z-Surveyor

firmware release UF00. This version is still backward compatible with older

versions of Z-Surveyor firmware. Commander and Download are compatible

with the Z-Surveyor, Z-12, GG-24 and  SCA-12 receivers.

2.	A new Windows version of the Pacific Crest Radio Setup program has been

added.

3.  	A more robust firmware update utility, specifically, options in the Z-12 are now better protected during the firmware update process.

4.  	The Handheld Transfer, Pacific Crest Radio, and RINEX utilities are now 32 bit versions.

Known Limitations and Incompatibilities

- Most functions can be made to work on Windows 3.1. The RINEX converter, Handheld Transfer, and Pacific Crest radio utilities will definitely not work on Windows 3.1

2.	The Load Parameters command in Commander supports only the Z-Surveyor. The

function is not available for other receivers.

3. 	Modem connection to remote receiver may be established only in Commander.

Download can then be started using the same Com port so as to share this

modem connection.

4. 	The firmware and options uploading functions require that the receiver be continuously

powered on during the loading process. Note that these functions will only work

with a direct connection to the receiver, not a remote connection.

5. 	Reliance data sets are not supported by Download. Use the Reliance Processor's

Transfer utility to download Reliance SCA-12 receiver data.