Firmware Release Notes

Date:	August 27, 2001

Product:  Ashtech Marine Reference Station

Subject:  RE00 Firmware Release

Number:  RS2001-01

Introduction:  RE00 is a no cost firmware upgrade to replace RD00 firmware in the Marine Reference Station.  A pdf supplement to the Reference Manual that covers new features is available in the Reference Manual/Marine Reference Station folder on our ftp site at ftp://ftp.ashtech.com.

Fixes/Enhancements:

- The smoothing count in the B-file, both from the receiver's external memory and from the real-time messages, kept resetting from 250 to 0. The Smooth count in MBEN messages and Bfiles will now increment to 200 and then stop while an SV maintains lock.

- The GPS week number was not updated for unhealthy SV's even if the “use unhealthy SV” flag was set to 'Y'.  This has been fixed.

New Features:

- The backlighting of the Z12R was designed to stay on continuously.  The LCD light is now defaulted to off. After a 2-minute period of no activity on the keypad, as with other Z12 products, the backlight will now go off. The user is able to change this setting by entering subcommand "101" which makes the backlight stay on all the time.  Subcommand "100" will return the receiver to default.

- RSIM messages 27, 28, 29 and 30 have been added.

- Support of RTCM messages 18, 19, and 22 has been added. This capability requires the “3” soft option to be installed. Contact your Ashtech dealer for additional information and pricing.

Known Problems:

- RTCM message types 18, 19 and 22 work with version 6J00 (RTCM version 2.1) but are not compatible with the other Z family receivers (RTCM version 2.2).

- RTCM message type 22 continues to be output even after being disabled. Power cycling will correct the problem. Only if messages 18/19 are enabled and then disabled, or with a full memory reset completed, will the output of type 22 message stop.

- Station ID 1023 is not accepted when entered through the screen.  If setting station ID using RSIM #6, there is no problem and the value 1023 is accepted.

Known Incompatibilities:

There are no known incompatibilities.