Firmware Release Notice				Survey and Mapping

Date:	October 5, 1999
Product: Z-12, Z-12 Sensor

Subject: Version 6J00 Firmware Release

Number: RZ9903

Introduction:

6J00 is a no cost firmware upgrade for the Z-12 and Z-12 Sensor. It can be downloaded from Ashtech FTP site ftp://ftp.ashtech.com. Self-extracting ZIP file Z12-6J00.exe in the z12 folder contains and SEN_6J00.exe in the Z12 Sensor folder contain the  firmware and loading instructions.

Fixes/Enhancements:

- NMEA and RAW data messages can be output at 2hz without the “Q” option.  With the “Q” option, NMEA and RAW data messages can be output at 5hz.

When a Z-12 is installed with a ‘Q’ option, the receiver can be switched to operate in 2Hz mode (default) or 5Hz mode. The mode switch can be done with receiver screen/keyboard interface.

Screen interface:   SCRN 4 / RCVR CTRL

|  | 01 | 23456789012 | 3456 | 78901 | 2345 | 67890 |  | 1234 | 56789 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | RECEIVER CONTROL MENU |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  | Z MODE | A |  |  |  |  |  |  |
| 3 |  | RANGER MODE | 0 |  |  |  |  |  |  |
| 4 |  | ACK/NAK FMT | $PASHR,ACK/NAK |  |  |  |  |  |  |
| 5 |  | CYCLE TIME | 0.50 (sec) |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |
| 7 | Press e to change |  |  |  |  |  |  |  |  |

CYCLE TIME can be changed between 0.50 sec (2Hz) or 0.20 sec (5Hz).

- Checksum has been added to all NMEA messages

- Added $PASHS,CPD,AUT,on/off set command to set auto differential mode in CPD and RTCM 18/19 mode.  Setting of this parameter is displayed by the $PASHQ,CPD,MOD query command, and by screen 4, phase dif/solution params screen.

- The range of the antenna slant is now 64.00 meters.  It was previously 6.4 meters.

- Cold and warm start times have been improved over 4J00.

- Firmware is GPS EOW and Y2K compliant.

- Changes in the OBN creation features were implemented to correct problem with missing vectors or incorrect vectors when using hybrid or TDS software.

- RZ will now work in RTCM 18/19 remote differential mode when dual frequency CPD/GLONASS corrections are received.

- Command $PASHS,SAV,Y is equivalent to subcommand 555 and command $PASHS,SAV,N is equivalent to subcommand 550.  $PASHS,SAV is still valid and equivalent to command $PASHS,SAV,Y.

New Commands/Features - (Descriptions are  in new Z12 manual that is available in PDF format on the FTP site with the firmware.  Those not in the manual are covered in the attached file.)

- Added ZDA NMEA message.

- Added following set commands: LTZ, SVS, INF.

- Added following query commands: INF,SVI, RTL, MEC.

Known Limitation or Problems:

- 1PPS output is not supported.

- Will stop computing a position when brought near the N or S pole.  The receiver will usually recover from this situation after moving several kilometers from the pole.  An internal memory reset is sometimes necessary to restore normal operation.

- When a new SV is used for the first time in a CPD solution with fast CPD off, the mode flag in the GGA message goes from 3 (fixed) to 2 (float) for one or two seconds.  There is no jump in the position solution when this occurs.

- UTC year field in the MEC response message is incorrect.

Known Incompatibilities:

- To get the correct antenna height information displayed in the S-file, data needs to be downloaded using Hose ver, 5.5.00.  WinPrism ver. 2.1 or RCS 3.2.  If earlier versions or Hose or WinPrism are used, data will tranfer properly, but the incorrect antenna height information will be displayed in the S-file.

New Z12 & Z12 Sensor Commands          	Survey and Mapping

- Date:		October 5, 1999
Product	Z12 and Z12 Sensor

#### INF: Set Session Information$PASHS,INF,c1,s1,s2,s3,s4,s5,f1,d1,d2,d3,d4

Sets a variety of session information parameters.

where the fields are as defined in Table 6.13.

| Table 6.13: INF Command Structure |  |  |
| --- | --- | --- |
| Field | Description | Range |
| c1 | session name | 1 alphanumeric char |
| s1 | receiver serial number | 3 alphanumeric char |
| s2 | antenna number, up to 3 ASCII characters | 3 alphanumeric char |
| s3 | month and day of the session (mmdd) | 01-12 month 01-31 day |
| s4 | operator identification | 3 alphanumeric char |
| s5 | user comment | up to 9 alphanumeric char |
| f1 | antenna height in meters | 0.0000-64.000 |
| d1 | dry temperature in degrees Celsius | -99 - +99 |
| d2 | wet temperature in degrees Celsius | -99 - +99 |
| d3 | relative humidity in percent | 0 - 99 |
| d4 | barometric pressure in millibars | 0 - 9999 |

Example: Set session parameters

$PASHS,INF,A,325,401,0313,DWK,test-Proj,1.456,65,60,65,1010

#### INF:  Query survey session parameters:

#### $PASHQ,INF,c

Query the survey session parameters, where c is the optional output port.

Example: Query session parameters to the current port.

$PASHQ,INF<Enter>

#### $PASHR,INF

The response message is in the form

$PASHR,INF,f1,d2,d3,d4,c5,d6,d7,s8,c9,s10,s11,d12,s13,s14,f15,d16,d17,d18,d19,
f20,d21,d22,d23,d24

where the fields are as described in Table 6.14

| Table 6.14: INF Response Structure |  |  |
| --- | --- | --- |
| Return Parameter | Description | Range |
| f1 | Data recording interval | 0.5 to 999.5 seconds |
| d2 | Minimum number of satellites for data recording | 0 to 9 |
| d3 | Satellite elevation angle mask for data recording | 0 to 90 degrees |
| d4 | Type data recorded | 0 or 2 |
| c5 | Recording data switch | Y or N |
| d6 | Minimum number of satellite for kinematic alarm. |  |
| d7 | Number of epochs to go for kinematic survey | 0 to 999 |
| s8 | Site name | 4 alphanumeric characters |
| c9 | Session name | 1 alphanumeric character |
| s10 | Receiver number | 3 alphanumeric characters |
| s11 | Antenna number | 3 alphanumeric characters |
| d12 | Month and day of session, mmdd | 0101 to 1231 |
| s13 | Operator identification | 3 alphanumeric characters |
| s14 | User comment | up to 9 alphanumeric characters |
| f15 | Antenna height in meters before data collection | 0.0000 to 6.4000 m |
| d16 | Dry temperature before data collection | -99 to +99_C |
| d17 | Wet temperature before data collection | 99 to +99_C |
| d18 | Relative humidity before data collection | 0 to 90% |
| d19 | Barometric pressure before data collection | 0 to 9999 millibars |
| f20 | Antenna height after data collection (meters) | 0.0000 - 64.000 |
| d21 | Dry temperature after data collection (degrees celsius) | ±99 |
| d22 | Wet temperature after data collection (degrees celsius) | ±99 |
| d23 | Relative humidity after data collection (percent) | 0 - 99 |
| d24 | Barometric pressure after data collection | 0 - 9999 |
| *cc | Checksum |  |

- SVI:  Requests  satellite information.

Query: $PASHQ,SVI

#### The response is in the format:

$PASHR,SVI,d1,d2,d3,d4,d5,d6,h7,d8,c9*hh

where Table 5.17 outlines the response format:

| Table 5.17: SVI Response Structure |  |  |
| --- | --- | --- |
| Parameter | Description | Range |
| d1 | Number of locked satellites, 0 - 24 | 2 digits (0-24) |
| d2 | Number of fields per satellite, 1 - 99 | 2 digits (1-99) |
| d3 | Satellite PRN number, 1 - 56 | 2 digits (1-56) |
| d4 | Azimuth angle, 0 - 360 | 3 digits (0-360) |
| d5 | Elevation angle, 0 - 90 | 2 digits (0-90) |
| d6 | signal to noise ratio, 0 - 999 | 3 digits (0-999) |
| h7 | Satellite health code, two digit hexadecimal | 2 hex (0-ff) |
| d8 | User range accuracy, UDRA, two digits | 2 digits (0-99) |
| c9 | "U" or "-", usable/not usable in position computation | 1 char |
| *hh | The hexadecimal ckecksum is computed by exclusive O-Ring all of the bytes in the message between, but not including, the $ and the *. The result is *hh where h is a hex character. | 0-9 and A-F |

- RTL:  Requests RTCM and the entered position information.

Query:  $PASHQ,RTL

Command:	$PASHR,RTL, f1, c2, f3, c4, f5, d6, c7, d8, c9, d10, d11,d12 *hh<CR><LF>

| Parameters | Description | Range |
| --- | --- | --- |
| f1 | User entered latitude | ddmm.mmmm |
| c2 | Latitude direction | ‘N’ or ‘S’ |
| f3 | User entered latitude | ddmm.mmmm |
| c4 | Longitude direction, | ‘E’ or ‘W’ |
| f5 | User entered altitude in meters | 0 - 9.999999 |
| d6 | Differential mode, | 0 = OFF 1 = BASE 2 = REMOTE |
| c7 | Autodifferential flag | ‘N’ = OFF ‘Y’ = ON |
| d8 | Station ID | 0 - 1023 |
| c9 | Differential port | ‘A’, ‘B’ , or ‘C’ |
| d10 | Index of baud rate for the differential port | 0 - 7 |
| d11 | Maximum age | 0 - 1199 |
| d12 | RTCM speed index | 0 - 8 |
| *hh | The hexadecimal ckecksum is computed by exclusive-ORing all of the bytes in the message between, but not including, the $ and the *. The result is *hh where h is a hex character. | 0-9 and A-F |

Example:		$PASHQ,RTL,B<CR><LF>

$PASHR,3722.39123,N,12159.83334,W,5.123456,1,Y,0777,C,5,200,5*FF<CR><LF>

- MEC:  Microcomputer Electronics Corporation support query

Command:	$PASHQ,MEC,[c1]<CR><LF>

(c1 -Response message is sent to the specified com. port A..D)

Return message:$PASHR,MEC,c1,c2,c3,c4,d1,f1,f2,f3,f4,f5,d2,d3,d4,f6,f7,f8,f9,

f10,f11,f12,f13,d5*h<CR><LF>

(The message is in the binary format)

| Return Parameters | Description | Range |
| --- | --- | --- |
| c1 | message status | bit0: UTC data valid bit1: Ionospheric data valid bit2: Date valid bit3: URA Flags valid |
| c2 | Day of the current day |  |
| c3 | Month of current date |  |
| c4 | Year of current date starting at 1980 |  |
| d1 | GPS week number | 0..999 |
|  | UTC Correction Data |  |
| f1 | A0 constant term of polynomial |  |
| f2 | A1 first order term of polynomial |  |
| f3 | Delta time due to leap second |  |
| f4 | Delta time due to leap second future |  |
| f5 | Reference time for UTC data |  |
| d2 | UTC reference week number |  |
| d3 | Week number leap second future |  |
| d4 | Day number |  |
| f6 | Ionosperic param Alpha0 |  |
| f7 | Ionosperic param Alpha1 |  |
| f8 | Ionosperic param Alpha2 |  |
| f9 | Ionosperic param Alpha3 |  |
| f10 | Ionosperic param Beta0 |  |
| f11 | Ionosperic param Beta1 |  |
| f12 | Ionosperic param Beta2 |  |
| f13 | Ionosperic param Beta3 |  |
| d5 | User Range Accuracy (URA) flag. Each bit is the URA Flag for one tracked satellite. MSbit is URA Flag for PRN 32. | 0 - good 1 - bad |
| h | Checksum. Computed by breaking the structure into 16 unsigned shorts, adding them together, and taking the least significant 16 bits of the result. |  |