ALH: Almanac Messages Received

$PASHQ,ALH,c

This command queries the receiver for the number of almanac messages that

have been received since the last power cycle, where c is the optional output port.

Using this query, a user can tell when all of the most recent almanac messages

have been received.

Example: Query the current port for the number of received almanac messages.

$PASHQ,ALH <Enter>

$PASHR,ALH

The response message is in the form:

$PASHR,ALH,d1,s1*cc <Enter>

Tabl e 6. 3. ALH Parameter Table

Parameter Significance Range

d1 Number of almanac messages received since power

up

0-32

s1 All almanac messages received

NO = not all almanacs have been received

OK = all almanacs received

NO

OK

*cc checksum