RELOADING  PM3 OS and software

Preparation of the SDCard:
1. Insert the SDCard in the card reader on your desktop. Format it in FAT32 
2. Copy in the root directory of your SDCard the following files:
   EBOOT140.TXT
   FOS216.TXT
   P011.TXT

3. And all the following directories:
   \ EBOOT140
   \ FOS216
   \ P011

4. Insert the SD card in the ProMark3 RTK.

Reloading the boot, OS f / w GPS and F / W
Turn on PM3
Double click the Utilities icon and launch AUTOLOADER
In the new window, select the file EBOOT.TXT and click OK 
At the end of the load, close the window "Loading ... Success"
Start again AUTOLOADER
Select the file FOS216.TXT and click OK 
At the end of the load, close the window "Loading ... Success": the PM3 reboot automatically.
Double click the Utilities icon and launch AUTOLOADER
In the new window, select the file and click OK P011.TXT
At the end of the load, close the window "Loading ... Success": PM3 will reboot automatically.
When PM3 has rebooted, do a Restore Factory Setting by pressing keys LOG IN + + MENU

Reloading of surveying application / MMapping
Copy the file "PM3EUV214.TXT" in the root directory of your SD card.
Copy the directory \PM3EUV214 (entire directory and not its content) on the SDCard
start AUTOLOADER, select the file PM3EUV214.TXT

reloading Fast Survey v2.1.3 (PM3 specific):
1. Insert the SDCard in the card reader on your desktop. Format it in FAT32 
2. Copy the file "Fast Survey.CAB" in the root directory of your SD card.
3. Copy the directory Grids (entire directory and not its content) on the SDCard
4. Insert the SD card in the ProMark3 RTK.
5. On ProMark3 RTK, double click the Utilities icon and run Install FAST Survey.
6. Make sure the box "FAST Survey" is checked in the window.
7. check (if any) all grids you want to use
8. Press the Install button and let the unit complete the installation.
9. At the end of the load, close the window "Loading ... Success" by clicking OK. The loading of the grid will starts
10. At the end of the load, close the window "Loading ... Success"

11. Do a reboot of PM3 in order to change the Desktop icon and display FAST Survey on the main menu.

validation of different RTK and Fast Survey Codes
Double-click the Utilities icon
Double-click on Unlock RTK option
Click on the bottom right of the screen to bring up the virtual keyboard
Enter the code corresponding to PM3 serial number
At the end of the process, close the window "RTK is unlocked with success ..." Click OK
Double-click on Unlock FAST Survey
Enter the code for the serial number of the PM3
At the end of the process, close the window "FAST Survey is unlocked with success ..." by clicking OK