# Procedure to set Aquarius/Sagitta in WAAS/EGNOS mode

- Switch ON the receiver.

- From the Main menu press F2 (DGNSS menu)

- Press F3 (Mode menu)

- Press F2 (New line).  Prompt jumps to WAAS line.

- Press F4 (>>>).  Prompt changes to STATION column.

- Select AUTO by pressing 

- Press F2 (INIT).  INIT window opens.

- Enter 1 or 2 WAAS PRN number(s) to track using number keypad.  (West Coast US, PRN = 134; East Coast US, PRN = 122; Western EGNOS, PRN = 120)

- Press F5 (OK) to validate.  Screen returns to MODE menu.

- Press F4 (>>>).  Prompt changes to USED column.

- Press  to select WAGPS.

- Press F5 (OK) to validate.  Screen returns to DGNSS menu.

- Press F1 () to return to the main screen.

- Press F3 (AUX)

- Press F5 (>>>) until INIT menu option appears.

- Press F2 (INIT)

- Press F5 (>>>) until DESEL menu option appears.

- Press F4 (DESEL).  Screen changes to DESELECT menu.

- Set Minimum elevation with the number keypad.

- Deselect WAAS PRN number (ex: 122 and 120).  Use arrow keys  and  to move the cursor.  (This deselection allows using satellite corrections, but not using the satellite pseudorange).

- Press F5 (OK) to validate the entry.

- Press F1 ().  Screen returns to INIT menu.

- Press F5 (>>>) until STATUS menu option appears.

- Press F2 (STATUS).  Verify that you are receiving the WAAS satellite(s).

Ex:  134  d/d  39  =15  250

Verify that the L1/L2 column displays a d/d and that the ELV column displays an = sign.

- Press F1 ().  Screen returns to AUX menu.

- Press F1 ().  Screen returns to Main menu.

- Verify the solution status on top of the screen (2 lines):

Ex:  June 05 2003  WGPS   Q.9

UTC  15:42:00  ****    1/3

WGPS = WAAS/EGNOS mode

Q.9 = Quality of the solution from 4 to 9; 9 is the best.