# Engineering Update ### Aquarius 01-02 ### Aquarius²11-12-22 ### Sagitta 01-02 ### 3011 ### July-2002 #### CONFIGPACK Software #### V3.33

TABLE of CONTENTS.

- Introduction

- Equipment concerned

- Compatibility

- Restriction

- Enhancements

- Necessary means for upgrading

- Step-by-step procedure

- Attached items

- INTRODUCTION

THALES NAVIGATION makes every effort to satisfy its clients by regularly releasing enhanced versions of its products. The object of this document is to provide all the instructions required for updating any equipment already in use in the field so that users can benefit from the new enhancements.

- EQUIPMENT CONCERNED

Product line	: Marine survey

Product	: Aquarius01/02-Aquarius²11/12/22-Sagitta01/02 - 3011

Sub-part	: N/A

Software	: 

Hardware	: 

- COMPATIBILITY

Before updating it is recommended to:

- Update PC  board with version (UCIMV20009) for Sagitta and Aquarius

Software with Navigation and HF/MF functionality.

- RESTRICTION

- New  Configuration generated with  CONFIGPACK  are not compatible with former CONFPACK used with 5000 and 6000 series (and reverse).

- ENHANCEMENTS

- Navigation functionally (Way points and routes).

- HF and MF reception parameters setting.

- EQUIPMENT REQUIRED

- A PC computer using WINDOWS95, 98,XP, Millennium or WINDOWS NT.

- STEP-BY-STEP PROCEDURE.

This software is available on ftp site. Address    ftp://ftp.ashtech.com/

or

- Introduce the CD software package in your PC.

- Click on “Install “CONFIGPACK 3.33” to start the set up.

- Follow screen instruction up to final set up.

- An internal configuration reset of your receiver is necessary after update. To do so, depress the scroll button during the start up procedure until to get “INTERNAL RESET IN PROGRESS”.

- Read the internal configuration of your receiver with CONFIGPACK.

- Start from this configuration to generate a new one.