# NR203 V4.1 ENGINEERING UPGRADE

# CONTENT

# Introduction

Please, find enclosed the new versions GPS203 and DIF03 for the  GPS203 board and  DIF03 board of the GPS receiver  AQUARIUS NR203. The GPS version 4.1 replace the version GPS203 V3.1 dated 23/07/1998. The DIF03 version 4.1  replace the version  DIF03 V3.0 dated 12/09/1997. The  version PWES1  V3.0 dated 12/09/97doesn't change.

After update, the NR203 configuration should be :

| Product | GPS board | DIFF board | Auxilary board |
| --- | --- | --- | --- |
| Mobile NR203 | Type : GPS03-15CX GPS203 V4.1 27/01/2000 | Type : DIF03 DIF03 V4.1 27/01/2000 | Type : PWES1 PWES1 V3.0 12/09/97 |

IMPORTANT : The versions GPS203 and DIF03 . To ensure the work of one of the boards, both have to be updated , version 4.1:

| - Process improvement of HF stations encryted dual-fréquencies - Process improvement of RTCM message type 3 - Process improvement of RTCM message type 16 - New variables : standard deviation of semi-major axis of error ellipse, standard deviation of semi-minor axis of error ellipse, and orientation of semi-major axis of error ellipse - New messages in the default configuration (message type NR103, NMEA 0183 with parity, $GPGST) - Implantation of decoding fonction for the DGPS SeaStar DP service from FUGRO |  |
| --- | --- |

# Delivery list

GPS203

| Description | Reference | file | size | date |
| --- | --- | --- | --- | --- |
| Binary of the GPS203 software for the programmation with a RS link | GPS203x V4.1 27/01/2000 | GPS203X.X | 983 156 octets | 27/01/00 04:01:00 |
| Document file for the use of the update software |  | GPS203.UPD | 53 octets | 27/01/00 04:01:00 |

DIF03

| Description | Référence | fichier | taille | date |
| --- | --- | --- | --- | --- |
| Binary of the DIF03 software for the programmation with a RS link | DIF03x V4.1 27/01/2000 | DIF03X.X | 185 742 octets | 27/01/00 04:01:00 |
| Binary of the DIF03 software for the programmation with a RS link |  | DIF03.UPD | 51 octets | 27/01/00 04:01:00 |

# Improvements

## New fonctions

### Process improvement of RTCM message  type 16

Until now, only RTCM SC104 messages type 16 for selected stations were accessible. An extension of the decoding allow you to store messages inside a unique message list of  8 messages. The process and the storage of  recognised message for a  selected station has been kept to use user variables as (UM1..UM6). We take care about the station ID for  the new process of the Diff display.

### New user variables (ellipse error for the point)

Three new variables: SDMA standard deviation of semi-major axis of error ellipse , SDMI standard deviation of semi-minor axis of error ellipse, and IE orientation of semi-major axis of error ellipse.

### Default configuration

#### Updating of the almanachs, iono, utc. and  9 new formated messages  (see the following )

| Ident | Port | Mode | Rate | Contents |
| --- | --- | --- | --- | --- |
| 8 | A | TIME | 1sec | Message type NR103 (début) QLY, ZDA |
| 9 | A | TIME | 1sec | Message type NR103 (2ème partie) GLL, VTG |
| 10 | A | TIME | 1sec | Message type NR103 (fin) SGD, SYS, ZEF |
| 11 | A | TIME | 1sec | Global Positioning System Fix Data – GPGGA position WGS84, précision 10-4 minutes, vith parity |
| 12 | I | TIME | 1sec | Global Positioning System Fix Data – GPGGA position WGS84, précision 10-4 minutes, with parity |
| 13 | I | TIME | 1sec | Course Over Ground and Ground Speed – GPVTG with parity |
| 14 | I | TIME | 1sec | Global Positioning System Fix Data – GPGGA position WGS84, précision 10-5 minutes, with parity |
| 15 | J | TIME | 1sec | Global Positioning System Fix Data – GPGGA position for navigation, précision 10-4 minutes, with parity |
| 16 | J | TIME | 1sec | GPS Pseudorange Noise Statistics – GPGST with parity |

## Corrections

### Process improvement of encryted HF stations HF dual frequency

Improvement at the limit of the coverage,  when you get disturbances.

### Process improvement for RTCM message type 3

Buffer problem with  two messages type 3 rate 0,6 secondes is solved.

### RTCM digital aquisition and influence on HF/Mf process.

On RTCM reception , there was a  loss of HF reception with serial port not properly configured . This has been corrected and tested on both port L and K.

# Software identification version

Command  TEST 1 on serial port :

| TYPE GPS203x V4.1 27/01/2000 GPS03 C2 314290 ID F0 STANDARD CONFIGURATION R203__MT V4.0 11/01/2000 DIF03x V4.1 27/01/2000 | <- GPS03 software version <- GPS03 board identification <- GPS03 board Identification Configuration <- DIF03 software Version |
| --- | --- |

# Implementation of the decoding function for the new DGPS SeaStar DP service (FUGRO)

This new firmware of the NR203 allows to decode the new Seastar DP service of  Fugro for the DP market.

For more information , please contact :

Fugro Seastar AS:

Address	PO Box 50, Røa, N-0701Oslo, Norway

Phone	+47 22 13 47 00

Fax	+47 22 13 46 46

e-mail	seastar@fugro.geoteam.no

- Web	www.seastar.co.uk

B - PROCEDURE

# Necessary EQUIPEMENT

- 1PC Windows 95

- 1 Software “Update 95” version 1.0 of 15 Avril 1997 with  its notice.

- 3 cables:

— 1 cable 9/9 M/F (délivered with the 203)

— 1 cable adaptateur SUB 15 M / SUB 9 F (see Annex 1)

— 1 cable adaptateur RS422M / RS232 F (see Annex Il)

- 1 floppy  1,44 M octets enclosing new files (V4.1) which contains described here under:

| DIF03 X | 185742 | 27/01/00 | 4 :10 | Binary of the DIF03 software for the programmation through RS link |
| --- | --- | --- | --- | --- |
| DIF03 UPD | 51 | 27/01/00 | 4 :10 | Document file for the use of update |
| GPS 203 X | 983156 | 27/01/00 | 4 :10 | Binary of the GPS203 software for the programmation through RS link |
| GPS 203 UPD | 53 | 27/01/00 | 4 :10 | Document file for the use of update |
| PWES1XX | 158319 | 12/09/97 | 3 :00 | Binary of the PWES1 software for the programmation through RS link |
| PWES1 UPD | 51 | 12/09/97 | 3 :00 | Document file for the use of update |

# INSTALLATION OF the “UPDATE 95”SOFTWARE

Read the following procedure beforesetting this software:

ATTENTION

This program runs under  a WINDOWS 95 environment. To run properly check the parameters of the following instruction:

— Select “start”> “settings”>

“control panel” with the key at the left bottom of your screen

— double click on “System” in the “control panel“.

— Click on the  “device manager"’tab

— double click on “Port (COM&LPT) “.

— double clic on serial port to be used (par example COM1).

— double clic on the “Port setting"tab

— Clik on “Avanced port setting... "tab.

In the dialog box

Click on "use FIFO buffer”

Put “receive buffer” on “high "

Put “transmit buffer” on “low”

— Click “0K” to close dialog box “advenced port setting”.

— Click “0K” to close dialog box “ports (COM&LPT)"

— Click “0K” to close dialog box “system “.

— close the box “control panel “.

Then following the setting procedure described in  the notice;

# UPDATING THE PWES1, DIFO3, GP5203

## Using the software

- First use of the software: transfer all files to the update 95 directory.

- Second use of the software: if you have already use this software to pass from the 1.0 version  to 3.0 version, you have to save in a directory called " buffer or other" all files corresponding to version 3.0 (DLFF 03.UPD, DLFO3X.S, DIFO3X.X, 0P5203.UPD, GPS2O3X.S, GPS2O3X.X, PWES1.UPD, PWESLX.S, PWES1X.X). Then, transfer all files from the 4.1 floppy to the directory UPDATE 95.

## Updating PWES1 board

BEWARE: the version PWES1 which ensures compatibility with new versions 4.1of the GPS203board and DIF203 board is still V3.O dated 12/09/97. Please upgrade only in case the V3.0 is not implemented .

- Link  PC to NR203 (PORT I). Run  software through RS422/RS232 connection cable.

- Follow what is written in the  notice:

— average transfer time: about. 2 minutes.

— the message “Reprogramming board PWES1"will appear on your screen during the transfer time.

## Updating differential board

- Link  the PC to receiver (port L 15 pins) with the appropriate cable , then, transfer the configuration diff with the help of “UPDATE”. Transfer average time: 2 à 3 minutes. No particular message on the screen, only the PWES1 board version.

## Updating GPS board

- Link the PC to receiver (PORT A) cable RS232, then, transfer  the GPS 203 configuration . Transfer average time: 11 to 13 minutes.

- Stop the receiver

- Start it again keeping the "enter " key on to take into account the default configuration. Then reload your own configuration..