The document specifies CSV format of files created by

MobileMapper Field application

# Introduction

When a user defines a GIS map in MobileMapper Field application, there is a possibility to create CSV (stands for Comma Separated Value) type of layer. The following types of attributes are possible in this case:

- Text

- Menu (note that when a user defines this tape of attribute, a *.mnu file is created in the same folder where *.csv file is stored. *.mnu file contains menu items entered by a user)

- Image

- Voice

Only Point type of features is possible to store into CSV layer.

# CSV format description

The format of the created *.csv file is the following:

  - The first string defines the coordinate system which has been specified by a user at the beginning (it is the same as the one presented in *.map file);

  - The second string contains attribute and coordinate names separated by a comma “,”. The order of attribute names in this string is defined by a user at the definition stage of CSV layer for attributes. Note that coordinate names are always present at the end of the string as X,Y,Z.

    - If the coordinate system is Latitude/Longitude type, then X is Longitude, Y is Latitude, Z is Height. Latitude and Longitude has DDD.DDDDDD format;

    - If the coordinate system is Plane type, then X is Easting, Y is Northing, Z is Height expressed in the units of the defined coordinate system.

  - The third and all further strings contain attributive and positional information for logged points in the field. This information is separated by a comma “,” and the order of attributive and positional information is matching the order specified in the second string (see above).

# Content examples of CSV files

Example 1: WGS84 coordinate system (Latitude/Longitude type):

COMPD_CS["WGS 84",GEOGCS["WGS 84",DATUM["WGS 84",SPHEROID["WGS 84",6378137,298.257223563] ,TOWGS84[0, 0, 0, 0, 0, 0, 0]],PRIMEM["Greenwich",0],UNIT["Degrees",0.0174532925199433],AXIS["Long",East],AXIS["Lat",North]],VERT_CS["",VERT_DATUM["Ellipsoid",2002],UNIT["Meters",1],AXIS["Height",Up]]]

Picture,Sound,my text1,my menu1,my text2,my menu2,X,Y,Z

\My Documents\img009.jpg,\My Documents\Sound5.wav,t1,1,t2,3,37.653714,55.708532,232.185000

\My Documents\img010.jpg,\My Documents\Sound6.wav,t1b,2,t2b,4,37.653876,55.708843,152.113667

The full set of files for WGS84 example is here:

Example 2: UTM coordinate system (plane type):

COMPD_CS["UTM/WGS 84/UTM zone 37N",PROJCS["",GEOGCS["WGS 84",DATUM["WGS 84",SPHEROID["WGS 84",6378137,298.257223563] ,TOWGS84[0, 0, 0, 0, 0, 0, 0]],PRIMEM["Greenwich",0],UNIT["Degrees",0.0174532925199433],AXIS["Long",East],AXIS["Lat",North]],PROJECTION["Transverse_Mercator"],PARAMETER["latitude_of_origin",0],PARAMETER["central_meridian",39],PARAMETER["scale_factor",0.9996],PARAMETER["false_easting",500000],PARAMETER["false_northing",0],UNIT["Meters",1],AXIS["East",East],AXIS["North",North]],VERT_CS["",VERT_DATUM["Ellipsoid",2002],UNIT["Meters",1],AXIS["Height",Up]]]

tttt,Picture,Sound,mymenu,X,Y,Z

my text,\My Documents\img011.jpg,\My Documents\Sound7.wav,c,415510.711328,6174513.062883,56.929334

ffffgggg,\My Documents\img012.jpg,\My Documents\Sound8.wav,d,415513.311382,6174521.923840,50.303334

The full set of files for UTM example is here: