Firmware Release Notes	MMCX

Date:		October 13th, 2011

Product:  	MobileMapper CX

Subject: 	Mobile Mapper CX Firmware Release

- Introduction

This document is the firmware release note of version E034 for MobileMapper CX

This version is intended to fix FNC#2718 (see note below)

- Upgrade procedure

- Copy baldur.exe and FeodoreAndPfl.bin files to MobileMapper CX receiver (to SD card, for example);

- Execute baldur.exe, select GPS1 port and press “Connect GPS” button;

- After several seconds, make sure you see “GPS connected at 4800” at the end of the output;

- Choose FeodoreAndPfl.bin (normally it should be selected automatically if both *.exe and *.bin files are located in the same folder) and press “Load” to upload the GPS FW;

- Wait for the end of GPS FW upload (a few minutes) and close baldur.exe application;

- Restart MobileMapper CX receiver manually;

- After reboot, make sure you have E034 for GPS F/W Version in Start\Settings\Control Panel\Magellan System Info\ menu

- Versions compatibility

From compatibility point of view, version E034 has no difference from its predecessor E033. It’s safe to replace version E033 with E034.

- New features

No new features introduced in E034

- Resolved Problems

- FNC #2718.  ProMark3 losing connection to satellites. Deeper investigation showed that PM3 may experience Baldur FW resets while tracking more than one SBAS satellite. Then it takes the receiver some time to restore tracking and satellites usage after reset. Engineering believes that MobileMapper CX could also be affected by this problem. Version E034 puts protection against this problem.

- Known issues

No new known issues in comparison to previous version.

- Recommendations

- It’s recommended to upgrade from E033 to E034 in case of any user complaints regarding loss of satellites in MobileMapper CX.