Firmware Release Notes					OEM Solutions

Date:		February 27, 2003

Product:	Magellan CP14 OEM Board and Sensor

Subject: 	Firmware Version KC00

Number:  CP2003_01

Introduction:

KC00 is a no-cost firmware upgrade for the Ashtech CP14 OEM receiver boards and sensors to replace KA00 CP14 firmware version.   KC00 can be downloaded from the firmware/CP14 folder on Ashtech FTP site ftp://ftp.magellangps.com.   The self-extracting ZIP file CP_KC00.ZIP for CP14 receiver board and sensor contains the KC00 firmware, loading instructions and these KC00 firmware release notes.

# Changes and Improvements:

- Quality indicator in the GGA message will indicate an invalid position when no position is available.

- Corrected problem in the MSG messages for Type 1 and 2 where the same satellite listed was occasionally listed twice with different data.

- The number of SV groups in the NMEA messages can now be changed from 14 to 12 to be compliant with NMEA standards by issuing the command $PASHS,GPS,CHN,12.  The number of SV groups is 14 by default.