# ADU3 Loading Instructions using the Universal Program Loader

NOTE:  The firmware will load at an inconsistent speed and will be accompanied by a number of beeps.  This is normal.  After all power cycles, wait for the second beep before trying to communicate with the receiver.

- Place ADU3 firmware in a folder on the hard drive.

- Connect I/O cableto port X (labeled “C” on some units) and power up receiver.

- Start Universal Program Loader

- Select Firmware of the receiver

- Browse to file folder where DA52.bin is located, select and open

- Click on Additional

- Click on Advanced

- “check” allow advanced settings before loading & click on OK

- Select correct PC COM port.

- Click on <Next>

- Under specify receiver type and mode of the receiver, select DG-14 for either the CP-14 or the DG-14.

- Leave Mode set to Normal

- Under Chose connection type, select Serial Cable

- Under settings select 115,200, 8, N, 1 and “check” the Use RTS/CTS Box.

- Click on OK

- Under select section(s) of firmware or options to load, select Main Section

- Click on <LOAD>.

- When finished, no not exit the UPL.  Power cycle the receiver.

- On the UPL program, click on <Back>.

- Under select section(s) of firmware or options to load, select Firmware of the inner receiver (inside the ADU3)

- Click on <Load>.  When finished, do not exit UPL.  Power cycle the receiver.

- Connect the computer to port Y (or D).

- On port Y, do steps 15 through 20.

- Following power cycle (step 20), click <Back> to the Program Loader: Settings screen.

- Move data cable from port Y (or D) to port A.

- Browse and select the nav.dat and bin.dat files and open.

- Click on <Next>

- Under specify Type and Mode of the Receiver, select the ADU3

- Under select section(s) of firmware or options to load, select Boot and Main sections

- Click on <LOAD>

- Follow directions on the screen.  When finished, firmware should be AC00,DA52 in the RIO string or AD00 in the RID string.