Firmware Release Notes				OEM Solutions

Date:		July 10, 2006

Product:	Magellan ZXW Sensor & ZXW Eurocard

Subject: 	Firmware Version ZG05

Number:  ZXW2006_01

Introduction:

ZG05 is a no cost firmware update for the ZXW Sensor and ZXW Eurocard.  WINZIP file ZXW_ZG05.ZIP contains the ZG05 firmware, loading instructions, and these f/w release notes.  High flash memory firmware can only be loaded into receivers with boot code 1A01.  Trying to load it into other receivers may damage the receiver requiring non-warranty repair.  The boot code is the last field prior to the check sum in response to query <$PASHQ,RID>.  ZG05 is not recommended for the Z Sensor or Z Eurocard; these units do not have high flash memory.

New Features or Changes:

- Improved time to first fix following a warm start.

- STI filter is available for all RTCM messages.

- The problem with the position jumps in case of the unhealthy satellite usage has been fixed.   Previously, if receiver used an ephemeries from previous visibility zone, and the satellite state changes from healthy to unhealthy between two visibility zones, but was not broadcast in the ephemeries, the bad measurements could result in position jumps.   The maximum ephemeries age has been changed from 10 hour to 4 hour.